Example 8:  Well-Posed one dimensional Horizontal Flow Problem

This example show how to set up and solve a well posed 1-D inverse problem. The problem is divided into two parts: first the forward model is used to generated observations at specific well sites. These observations will then be used in an inverse model to predict the domain conditions of the forward model. The instructions for this example are provided as a list of steps with accompanying screen captures.

Part 1: Generating observations well data using:

- New Project

- Start a new project by selecting File then New Project.

Define the initial grid dimensions (you can edit this later).

- Change the value for the number of rows to 1

- Change the value for the number of columns to 10.

- Change the row and column spacing to 1.

- Select OK

You should have been advanced to the main VSAFT2 window.  Your screen should look like the one below.

For this example we will accept the grid without editing the row or column spacing or adding additional rows or columns.

- Select Accept Default

- Select Next to advance to the “problem definition” tab.

- Problem Definition

- Enter a title in the “TITLE” box.  This is for record keeping purposes and to assist in remembering the details of the model.  Use a descriptive title.

- Use the “Problem type” drop down menu to select Steady State Flow

- Use the “Flow type” drop-down menu to select Horizontal Plane

- Choose model type as Forward

- Select Next to continue to the simulation control tab.

- Simulation Control

- Select the Lumped storage scheme.

- Select the Linear under the “variation of conductivity within an Element” section.

- Set the Pressure head tolerance to 0.000001

- Select Next to continue to the Material tab.

- Materials

This example uses random material.

- Click Define in the “Zone Material Properties” section

- Choose Heterogenous for representing random material.

- In the Hydraulic properties box select Saturated Flow properties.

Enter the X and Y correlation scale and the variance for Ksx, Ksy.

- X correlation scale = 1

- Y correlation scale = 1

- Variance (ln) = 0.2

- Mean (ln) = 0

- Select OK

- Select Next to continue to the “Initial condition” tab

- Initial conditions

Set the initial hydraulic head

- Select Pressure Head

- Enter the initial hydraulic head of 100.0 in the “value” box.

- Select Apply.

- Select Next to continue to the “initial condition” tab.

- Boundary

Here we will set the boundary conditions: the left side as a prescribed flux and the right side as a prescribed pressure.

- Select Add from the “boundary definition window”.

- With the mouse select the beginning and the end of the boundary on the left of the column.

- Also select the midpoint on the boundary so that VSAFT2 knows which direction along the boundary you wish to define.  This step is only required for the first boundary.

Once the boundary is defined a pop-up window will appear to enter the boundary values.

- Select flux and enter the value of 1.0.

- Select OK

Add another boundary condition to the right side of the domain.

- Select Add from the “boundary definition window” and define the right prescribed head boundary.

- With the mouse select the beginning and the end of the boundary on the right.

- Once the boundary is defined a pop-up window will appear to enter the boundary values

- Select Pres. Head and enter the value of 100.

- Select OK

- Select Next to continue to the “Source/observation” tab.

- Source/observation

We will not need to add any sources because we are describing natural flow condition.

Assign observation wells at each node on the bottom lines except the constant head boundary. These observation wells are shown with circles.

- Select Add under the “stress definition” section.

- Click the bottom part of the domain and create and add 10 observation wells totally.

- Name each well.

- Select Next.

- Output Control

- Make sure the Create output file box is checked.

- Leave the other settings as defaults.

- Choose finish save as to save the project.

- Save the project as Forward_8.

- Running VASFT2

- Select RUN and then VSAFT2

- When finished, the screen will look like this.

- Plotting the hydraulic Conductivity

- They hydraulic conductivity can be plotted by selecting View->Contour->Hydraulic Property Fields.

The generated field should be similar to the graph below.

Part 2: Using well data from part one in an inverse model:

- Problem Definition

- Select tab Problem Definition.

- Check Inverse in the model type.

- Then select the Materials tab.

- Material

- Select the Material tab.

- Click Define in the “Zone material” properties section. A window, like the one shown below will open.

- Under the “Estimate” section select the checkbox next to Ksx. Since we are modeling a steady state saturated flow problem, we can only estimate Ksx.

- Make sure the other parameters not being estimated from the part 1 are the same (they should be).

- Select OK.

- Now select the Source.observation tab.

- Source, observation

Now we can import the observed head generated by the synthetic test.

- Click import observations in the “Stress Definition” area.

- Find the location of the file O-FwdObs.dat located in the “Forward_8” folder generated from part 1.

- Select Open to import the data.

You can check whether the data is correct or not.

- Select Edit Well under the “Sources/Observation Definition” section and then click any of the observation well.

- A new window will open. Select the Observation tab and notice the fields are now populated with observational data from the forward model in Part 1.

- Select Ok.

- Select Next to continue to “Output Control” tab.

- Output Control

- Make sure the Create output file box is checked.

- Select Import Reference Field and a popup box will be displayed.

- Find the location of the file, material.dat located in the “Forward_8” folder generated from part 1.

- Select Open to import the data. This will allow us to compare data from the forward problem to the results generated in the inverse problem.

- leave the other defaults

- Click Next to continue to the “Criteria” tab.

- Criteria

The Criteria tab is used to define the parameters used for the parameter estimation.

- For this example, we will keep all the parameters as default.

- Choose Finish Save as.

- Save the file as Inverse_8

- Running VASFT2

- Select RUN and then VSAFT2

The program will run SSLE to estimate the unknown Ks values.

- View results

To view the estimated hydraulic conductivity for comparison with part 1:

- Select View->Inverse->Estimated Fields->Saturated Hydraulic Conductivity(Ks)

By default the plot will show the first iteration of the solver as the Pert. ln(K)

We need to advance the graph to the final iteration of the solver.

- Select Animate->Zones

- When the “Animate Zones” dialog box is displayed, select the End Zone drop down menu and select the last iteration in the list.

- Then select Animate.

Next we must make change ln(K) to K for comparison to our results in part one.

- Select the box, labeled … next to “Contour”

- When the “Contour & Multi-Coloring Details” dialogue box is displayed, select the dropdown box to change the variable being displayed. Select K.

You will get a hydraulic conductivity plot very similar to part 1 as seen below. If the scale is different than the graph in part one you can adjust it manually.

Another useful performance metric for the inverse model is a comparison of the pressure head simulated at the well sites in the domain to the pressure head observed from the forward model.

- This can be created by selecting View->Inverse->Performance->Scatter Plot->Head

By default the comparison of the observed pressure head to the simulated pressure head will show the first iteration of the solver. For each iteration the data solver will better fit the center black line shown below.

- To display the last iteration calculated select Animate->Mappings

- Once the Animate Mappings dialog box is displayed. Select the last iteration produced.

The solver finds solutions by minimizing the difference between the simulated variables and measured variables at the observation wells. Pressure head is one of the variables the program will minimize to achieve an R^2 value of 1.

- Comparison of K values from Forward and Inverse problem

An X-Y plot comparing the generated hydraulic conductivity values (K estimate) to the hydraulic conductivity values of the forward problem (K reference) are useful for determining the accuracy of the estimated hydraulic conductivity.

- To generate an X-Y plot open a new window of Tecplot 360

- Select File->Load Data File(s)…

- A “Select Import Format” dialog box will be displayed. Select, the default Tecplot Data Loader then select OK.

- Browse to the “Inverse_8” folder and select O-scatterplot_Ks.dat file; then select Open.

- The “Select Initial Plot” dialog box will be shown. For the “Initial Plot Type” choose XY Line then select OK.

- Select the Symbols check box to add symbols to every data point in the graph.

- Next click Mapping Style in the upper left hand corner of the “Tecplot” window to edit the axis and data being displayed.

- When the “Mapping Style” dialog box is displayed, select the Map Show button to change the iteration being displayed. We want to display the results of the final integration only.

- Adjust the parameters in the “Mapping Style” dialog box until your graph looks nice.

- The axes can be adjusted by selecting Plot->Axis… in the menu bar. Adjust both axes until all data is shown. When you are finished your graph should look something like the one below.

Because the data is well constrained, the inverse solver was able to matches the hydraulic conductivity data from the forward problem almost perfectly producing a straight line.