## Example 11: Hydraulic tomography for a layered system

This example shows how to set up a hydraulic tomography test with 5 pumping tests for a 5-layered aquifer.

Introduction to the problem:

In this example we will simulate groundwater flow in the North Campus Research Site at the University of Waterloo. Intensive field measurement of hydraulic conductivity, pumping test and 3D hydraulic conductivity analysis has been conducted recently. Using data collected at the NCRS site, this simplified 2-D example will explore some advanced topics of using VSAFT2 to conduct hydraulic tomography analysis.

We will first run forward transient simulation, then run the steady-state inverse model. Spatial and temporal units are assumed to be meter and day respectively.

We are using late time data from transient tests. So the procedure will be slightly different from example 10. We will not use the “generate synthetic data” option in inverse modeling, which is essentially forward modeling. Instead, we will run 5 transient forward tests and then retrieve their outputs for inverse modeling. In other words, we need to create 5 forward simulation projects and 1 inverse modeling project in this exercise.

- New Project

Start a new project.

Select File then New Project

Define the initial grid dimensions (you can edit this later).

Change the value for the number of rows to 52

Change the value for the number of columns to 18.

Change the row spacing to 0.25.

Change the column spacing to 2.5.

Select OK

You should have been advanced to the main VSAFT2 window.  Your screen should look like the one on the right.

For this example we will accept the grid without editing the row or column spacing or adding addition rows or columns.

Select Accept Default

You will be advanced to the problem definition tab.

- Problem Definition.

Enter a title in the TITLE box.  This is for record keeping purposes and to assist in remembering the details of the model.  Use a descriptive title.

Use the Problem type drop down menu to select Transient Flow

Use the Flow type drop-down menu to select Vertical Plane

Choose model type Forward for forward model.

Select Next to continue to the simulation control tab.

- Simulation Control

Select the Diagonal Lumping storage scheme.

Select the Constant variation of conductivity within an Element.

Set the Pressure head tolerance to 0.0001

Select Next to continue to the time steps tab.

- Time steps

Enter the Time Step parameters

Set the initial value of time step to 0.001.

Set the maximum allowable value of time step to 0.4.

Set the multiplier of time step to 1.05.

Set the maximum simulation time to 100.

Set the maximum number of reductions to 10

Select NEXT to continue to the materials tab.

- Materials

This example uses 5 layers of random material.

Set the total number of zones to 5.

Set the working zones to 5. Select Define with Polygon.

Enclose the lowest 4 m of the domain by hitting (x,z) = (0, 0), (0, 4), (45, 4), (45, 0) and (0, 0)

Similarly, set the working zones to 4. Continue to select Define with Polygon.

Enclose the 1.75 m above zone 5 by hitting (x,z) = (0, 4), (0, 5.75), (45, 5.75), (45, 4) and (0, 4)

Similarly, set the working zones to 3. Continue to select Define with Polygon.

Enclose the 2 m above zone 4 by hitting (x,z) = (0, 5.75), (0, 7.75), (45, 7.75), (45, 5.75) and (0, 5.75)

Similarly, set the working zones to 2. Continue to select Define with Polygon.

Enclose the 1.5 m above zone 3 by hitting (x,z) = (0, 7.75), (0, 9.25), (45, 9.25), (45, 7.75) and (0, 7.75)

The remaining domain is considered as zone 1 by default. The GUI should appear as the following:

Click Define to enter the material properties

Choose Hetergeneous for representing random material.

Enter the X and Y correlation scale and the variance for Ksx, Ksy (use the same inputs to assume isotrophy) and Ss. Variance of Ss is zero because we assume a homogeneous Ss field in each of the zones. Make sure you have input the values for all the zones. By the way, you could directly copy and paste the value into the material table.

| Zone |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Mean(ln) | Var(ln) | X Corr. | Y Corr. |
|  | Ksx | -3.5823 | 2.591 | 10 | 3 |
|  | Ksy | -3.5823 | 2.591 | 10 | 3 |
|  | n | -0.91629 | 0 | 1 | 1 |
|  | Ss | -5.3602 | 0 | 1 | 1 |
| 2 |  |  |  |  |  |
|  | Ksx | 0.1719 | 0.1316 | 10 | 0.5 |
|  | Ksy | 0.1719 | 0.1316 | 10 | 0.5 |
|  | n | -0.91629 | 0 | 1 | 1 |
|  | Ss | -7.6628 | 0 | 1 | 1 |
| 3 |  |  |  |  |  |
|  | Ksx | -5.0301 | 1.0205 | 10 | 0.5 |
|  | Ksy | -5.0301 | 1.0205 | 10 | 0.5 |
|  | n | -0.91629 | 0 | 1 | 1 |
|  | Ss | -5.3602 | 0 | 1 | 1 |
| 4 |  |  |  |  |  |
|  | Ksx | -1.6535 | 4.6766 | 10 | 0.5 |
|  | Ksy | -1.6535 | 4.6766 | 10 | 0.5 |
|  | n | -0.91629 | 0 | 1 | 1 |
|  | Ss | -7.6628 | 0 | 1 | 1 |
| 5 |  |  |  |  |  |
|  | Ksx | -6.7617 | 1.9769 | 10 | 3 |
|  | Ksy | -6.7617 | 1.9769 | 10 | 3 |
|  | n | -0.91629 | 0 | 1 | 1 |
|  | Ss | -5.3602 | 0 | 1 | 1 |

Select OK

To view the random hydraulic conductivity field generated, select First Hydraulic Conductivity  in the solution domain list and then select view with tecplot. In tecplot, select a proper contour range and use exponential scale for better visualization. Also adjust the x, y axes and change the variable name. We can clearly see the hydraulic conductivity field manifests as a 5-layer system.

Select Next to continue to the Initial condition tab

- Initial conditions

Set the initial hydraulic head

Select TOTAL Head

Enter the initial hydraulic head of 100.0 in the value box.

Select Apply.

Select Next to continue to the initial condition tab.

- Boundary

Set the boundary conditions.

Adding Boundaries

Select Add from the boundary definition window and define the left and right boundary both as prescribed Total Head.

With the mouse select the beginning and the end of the first boundary.  Also select the mid point on the boundary so that VSAFT2 knows which direction along the boundary you wish to define.  This step is only required for the first boundary.

Once the boundary is defined a pop-up window will appear to enter the boundary values

Select prescribed Total Head and enter the value of 100.0.

Select OK

Select Add from the boundary definition window and define the right prescribed head boundary.

With the mouse select the beginning and the end of the first boundary.

Once the boundary is defined a pop-up window will appear to enter the boundary values

Select Prescribed Total Head and enter the value of 100.

Select OK

Select Next to continue

- Source, observation

Set pumping rate as -2 at preferred node by selecting add and click at the node.

Selecting Edit Well to specify the well properties.

Exclude the pumping well as one of the observation well by removing the check of Active box in Observation section.

Add observation by first selecting the add icon and then clicking at the nodes. Zooming in using the “plus” icon may help.

Select Next

- Output Control

Select desired method of printout time. Because we want to use late time data to do the transient HT, thus, choose user specified then assign some late time  into the table.

Choose finish to save the project as tutorial-1 in the directory that you desire.

- Running VSAFT2

Select RUN and then VSAFT2

When running the screen will look like this.

Select Stop to return to VSAFT2

- Viewing results

Click VIEW and HYDROGRAPH in VSAFT2 and hydrographs will show up on Tecplot. V2 corresponds to the hydrograph at flow inspection #1, V3 for #2 and so on.

To retrieve head value at the end of the simulation (i.e. 100 days), click Data and Probe at in Tecplot. 
Set X=100 and hit Do Probe. All the values at time= 100 days will show up. Of course, you can do the same at other times.

Click VIEW and CONTOUR in VSAFT2 and contour maps will show up on another Tecplot window. Click on the icon next to Contour in Tecplot and select H. An H Contour will show up. If desired, check on click on the “legend” tab and check “show legend”

Since this is a transient simulation, it is critical to know how the head field evolves over time. We may visualize the above using animations. To do this in Tecplot, select Data and then Edit time strands. Click Apply and Close. Next, click Insert and Text and click on somewhere above the rectangular domain. Type time = &(SOLUTIONTIME)  days in the enter text string box and hit close. Finally, click on the play button on the left panel. The animation will play and the corresponding time will appear as the animation runs.

- Repeating forward simulation with a different pumping location

Keep everything equal, go to the SOURCES tab. Delete the source at element 619

Add a pump at element 495 and set pumping rate as -0.5.

Choose finish to save the project as tutorial-2 in the directory that you desire. Run the simulation.

Do the same for the remaining 3 tests using values in the table below.

- Pumping rates are in m2/day and are in brackets

- Pumping tests in each zones are conducted at the same depth

- Element number are in parentheses

End of Transient Forward Simulations

(We are using late time data from transient tests. So the procedure will be slightly different from example 10)

- Grid

From the previous steps, you should have created forward simulations in projects tutorial-1, tutorial-2, tutorial-3, tutorial-4 and tutorial-5. Now we will create a new project tutorial-HT to use the above results for inverse modeling (i.e. hydraulic tomography).

Open a new project in VSAFT2. Choose steady state and inverse model. Do NOT check the generate synthetic data box.

Define the initial grid dimensions (you can edit this later).

Change the value for the number of rows to 52

Change the value for the number of columns to 18.

Change the row spacing to 0.25.

Change the column spacing to 2.5.

Select OK

- Problem Definition

Select steady state flow. (We will use late-time head observations to estimate K only)

Again, the problem is in the vertical plane (recall the layered system in the forward simulations)

We will use SSLE (sequential successive linear estimator (Yeh and Liu, 2000))

Do NOT check the box for “generate synthetic data” (which is essentially forward simulation)

- Simulation Control

Keep everything as Default

- Material

Select tab Material, click Define. It will pop up the windows like the one shown on the right. Since it is a steady state saturated flow problem, we can only estimate Ksx. Select OK.

All the parameters for random field you used in the simulation are already remembered by the program. Click OK to import other materials which are not assigned.

Input mean(ln) as -1.4662

Input variance(ln) as 0.3*

Input x correlation as 10

Input y correlation as 0.75

*This is the initial guess for the variance of the estimated K field (which should be a small value of 0.2 or 0.3 and it should not the inversion result much)

Find the location of your project, chose the folder forward (for the example shown here, it is C:\tutorial-1\foward), click file material.dat to import the other materials. Select Open.

- Source, observation

Now we can import the observed head generated by the synthetic test.

Choose tab Source, observation.

Change total Number of Stress to 5

Add source/sink and observation as in tutorial-1, …, tutorial-5 by changing current stress as 1,…,5.

To edit pumping rate, select edit in source/sink. Then click on the pumping location. An input window will pop up. Remember to repeat the same procedure for all 1*5=5 pumping locations

For each of the tests, edit the observed head at each of the observation locations. Use the late-time (i.e. 100 days) head data in tutorial-1,…, tutorial-5 as the steady-state head. To edit, select edit in observation definition. Then click on the observation location. An input window will pop up. Remember to repeat the same procedure for all 18*5=90 pumping locations

Or, we can import data from

- Criteria

Choose tab Criteria. Define the parameters used for the parameter estimation.

Max. No. of iterations =50

Max. No. of loops = 1

Stabilizer = 5.0

Converge variance change = 0.0001

Converge pressure = 0.0001

SLE method: Simultaneous SLE

Choose Finish.

Choose Run and VSAFT2.

The program will run SSLE to estimate the unknown Ks values. When it reaches convergence, click Stop.

- View results

To view the property distribution derived from the inverse solution in TECPLOT

Select View->Properties XY plot. It will plot the comparison between the estimated and true properties.

Select the true material file as tutorial-1 -> forward -> material

Tecplot will pop up. Plot on loglog scales with the same ranges

The above is the K-scatter plot. There are 936 points because there are 936 elements in the domain for this example. If the estimate is perfect, all the points will lie on the 45-degree line. We can get better results by playing around with the convergence criteria. Now you see the points are scattering around the 45-degree line. So VSAFT2 is giving the best unbiased estimate of the K field.

Next, select View->results. It will plot the comparison between the simulated and observed head.

The above head scattered plot plots the head values input (from the forward simulations) and compare with the simulated head response using the K estimate. All the points lie very close to the 45-degree line. That means the estimated K field is very well conditioned by the observed head data. In other words, VSAFT2 has almost utilized all the head information available to give a best estimate of the K field.

Finally, select View->Properties Distribution. It will plot the contour map of the estimated K field.

You may compare it with the true K field (the one we used for forward simulations in tutorial-1, …, tutorial-5). Make sure the two plots have the same ranges on the axes and the same contour intervals.

This is the end of this example and I hope you enjoyed it. If you have any questions, please feel free to contact Michael Tso, graduate research assistant at Dr. Yeh’s group at the University of Arizona, at cmtso@email.arizona.edu