## Example 9:  Ill-Posed one dimensional Horizontal Flow Problem

This example shows how to set up and solve an ill-posed 1-D inverse problem. It is similar with example 8 except less observation wells are used.

Part 1: Generating observations well data:

- New Project

- Start a new project by selecting File then New Project.

Define the initial grid dimensions (you can edit this later).

- Change the value for the number of rows to 1.

- Change the value for the number of columns to 10.

- Change the row and column spacing to 1.

- Select OK.

You should have been advanced to the main VSAFT2 window.  Your screen should look like the one below.

For this example we will accept the grid without editing the row or column spacing or adding additional rows or columns.

- Select Accept Default

- Select Next to advance to the problem definition tab.

- Problem Definition

- Enter a title in the TITLE box such as Problem_9_Ill_Posed.  This is for record keeping purposes and to assist in remembering the details of the model.  Use a descriptive title.

- Use the Problem type drop down menu to select Steady State Flow

- Use the Flow type drop-down menu to select Horizontal Plane

- Choose model type as Forward.

- Select Next to continue to the simulation control tab.

- Simulation Control

- Select the Lumped storage scheme.

- Select the Linear for the variation of conductivity within an Element.

- Set the Pressure head tolerance to 0.000001

- Select Next to continue to the Material tab.

- Materials

This example uses random material.

- Click Define to enter the material properties

- Choose Heterogeneous for representing random material.

- In the Hydraulic properties box select Saturated Flow properties.

Enter the X and Y correlation scale and the variance for Ksx, Ksy.

- X correlation scale = 1

- Y correlation scale = 1

- Variance (ln) = 0.2

- Mean (ln) = 0

- Select OK

- Select Next to continue to the Initial condition tab

- Initial conditions

Set the initial hydraulic head

- Select Pressure Head

- Enter the initial hydraulic head of 100.0 in the value box.

- Select Apply.

- Select Next to continue to the initial condition tab.

- Boundary

Here we will set the boundary conditions: the left side as a prescribed flux and the right side as a prescribed pressure.

- Click Add in the Boundary Definition area.

- With the mouse select the beginning and the end of the boundary on the left of the column.  Also select the midpoint on the boundary so that VSAFT2 knows which direction along the boundary you wish to define.  This step is only required for the first boundary.

Once the boundary is defined a pop-up window will appear so the boundary values can be inputted.

- Select flux and enter the value of 1.0.

- Select OK

Add another boundary condition to the right side of the domain.

- Select Add from the boundary definition window and define the right side as a prescribed head boundary.

- With the mouse, select the beginning and the end of the boundary on the right.

- Once the boundary is defined, a pop-up window will be displayed

- Select Pres. Head and enter the value of 100 in the pop-up window.

- Select OK

- Select Next to continue to the Source/Observation tab.

- Source/observation

We will not need to add any sources because we are describing natural flow condition.

Add observation wells:

- Select Add under the stress definition section.

- Click the bottom part of the domain and create 3 observation wells at the nodes shown below.

- Name each well.

- Select Next.

- Output Control

- Make sure the Create output file box is checked.

- Leave the other defaults.

- Choose finish save as to save the project.

- Save the project as Forward_9

- Running VASFT2

- Select RUN and then VSAFT2

- When finished the screen should look like the picture below.

- Plotting the hydraulic Conductivity

- They hydraulic conductivity can be plotted by selecting View->Contour->Hydraulic Property Fields.

The generated field should be similar to the graph below.

Part 2: Using well data generated from part one in an inverse model:

- Problem Definition

- Select tab Problem Definition.

- Check Inverse in the model type.

- Then select the Materials tab.

- Material

- Select the Material tab.

- Click Define in the “Zone Material Properties” section. A window, like the one shown on the right will open.

- Under the “Estimate” section select the checkbox next to Ksx. Since we are modeling a steady state saturated flow problem, we can only estimate Ksx.

- Make sure the parameters not being estimated from the part 1 are the same (they should be).

- Select OK.

- Now select the Source.observation tab.

- Source.observation

Now we can import the observed head generated by the synthetic test.

- Choose tab Source.observation.

- Click Import Observations in the “Stress Definition” area.

- Find the location of the file FwdObs.dat located in the Forward_9 folder generated from part 1.

- Select Open to import the data.

You can check whether the data is correct or not.

- Select Edit Well under the “Sources/Observation Definition” section and then click any of the observation well.

- A new window will open. Select the Observation tab and notice the fields are now populated with observational data from the forward model in Part 1.

- Select Ok.

- Select Next to continue to “Output Control” tab.

- Output Control

- Make sure the Create output file box is checked.

- Select Import Reference Field and a popup box will be displayed.

- Find the location of the file, material.dat located in the “Forward_9” folder generated from part 1.

- Select Open to import the data. This will allow us to compare data from the forward problem to the results generated in the inverse problem.

- leave the other defaults

- Click Next to continue to the “Criteria” tab.

- Criteria

The “Criteria” tab is used to define the parameters used for the parameter estimation.

- For this example, we will keep all the parameters as default.

- Choose Finish Save as.

- Save the file as Inverse_9

- Running VASFT2

- Select RUN and then VSAFT2

The program will run SSLE to estimate the unknown Ks values.

View results:

To view the property distribution derived from the inverse solution in TECPLOT

- Select View->Inverse->Estimated Fields->Saturated Hydraulic Conductivity(Ks)

After adjusting the scale and iterations described in Example 8 the plot of the “Estimated Field” for hydraulic Conductivity will look like this:

Following the procedure described in “Example_8”, we can produce an X-Y plot comparing the hydraulic conductivity at each observation well calculated by the inverse solver and the actual hydraulic conductivity generated by the forward problem.

Notice, like the two hydraulic conductivity field maps produced above; there is a large variation between the estimated hydraulic conductivity and the reference hydraulic conductivity do the lack of information used in the inverse estimation. Notice that this result is produced despite the solver converging on a solution that perfectly matches the pressure head data in our three wells as seen below.

An Ill-posed problem often has multiple solutions due to a lack of knowledge about the system. Some solutions may be substantially different from the actual system.